55-60kn Torque head - Spec Sheet

Specification:

The 6000X has been designed for installing screw piles. The torque gauge fitted allows the user to monitor the installation torque as the piles are installed. The 8 bolt flange output allows a variety of adaptors to be fitted.

The 6000X torque head is designed to fit 8-30t excavators. This head will develop 60,000Nm torque at working pressures stated below-

Technical Information Specifications

- Max Pressure 300 Bar

- Max Flow 150l/min

- Max Speed 7rpm

- Net Weight 480kg without cradle

- Body Width 470mm

- Powerhead length (from base to centre of pin) 1300mm

- 2no Hydraulic hoses have ¾ inch BSP female fittings.

If you require any further technical information on the drive unit, please feel free to contact us on Info@ja-css.com